Q40 Fencing & Gates Specification

To be read in conjunction with Preliminaries/General conditions

Description: 		Steel turnstile temporary access gates

Standard:	BS1722-14:2017
BS EN 1991
CE Approved Force tested to 1200 Newtons

Manufacturer: 		CLD Fencing Systems  info@cld-systems.com

Tel. 01270 764751 www.cld-systems.com

Product Reference:	CLD FenceSafe PedMaster Turnstile System

Description:	Mild or stainless steel turnstile access gates identical manufacturing standards to permanent turnstile but with self-contained weighted base with lifting point for easy installation.

Height: 			1280 x 1400 x 1790mm

Usage: 			Indoor/Outdoor

Power Supply:		Single Phase 240Volt 50Hz 5 Amps

Arm size: 		42mm Tube with capped ends

Rotor size:		76mm Tube with Bottom Support

Finish:			Polyester powder-coated to 60-80 microns or 160-200 microns with marine grade and metallics. Any RAL colour available

Power fail modes:	Fail Safe or Fail Secure

Panel/Gate installation:	Self-weighted Base with Lifting Points

Flow rate:		15 to 22 Person per minute

Duty cycle:		100% Continuous Duty Rating

Unit weight:		230kg Approx

Side frame size:		33mm Tube Welded to base

Configuration:		Bi-directional or Single direction

Additional extras:	Can support additional Access control methods.

Rotor assemblies:	90 or 120 Degree rotor assemblies

Locking:	Lockable electro mechanical modules enabled in Fail-Open of Fail-Close cases. Can be manually unlocked by means of a key when required.

Warranty: 		Minimum of 12 months against manufacturing defects

Notes:	All dimensions are nominal, for further information please contact CLD Fencing Systems.

Information is correct at time of production. As part of our policy of continuous improvement, we reserve the right to alter product specification without notice.